This document tries to describe why we do what we do.

As we grow and start testing new techniques, please continue to update and expand this document.

If unsure, keep the new content highlighted so it can be fact checked by one of the PIs.

Here is an example of the layout, feel free to improve

| PATHWAY BEING TESTED |  |  |  |
| --- | --- | --- | --- |
| Measure | How | Reflects | References |
| Name of testing technique Duration of testing (if all goes well!) | Describe how to perform the technique Describe any alternatives GREEN = what our lab currently does (however, past studies might have used different methods listed) YELLOW = still updating… | What do the responses to this testing technique tell us? Define/describe in detail. Is there a mechanism behind the technique? | Who ‘created’ the technique? “Godfather” List papers that describe the technique very well NOTE: Please include full ref and add paper to the folder for future reference. If you see a reference incomplete, please complete it. |

| CORTICOSPINAL EXCITABILITY |  |  |  |
| --- | --- | --- | --- |
| Measure | How | Reflects | References |
| MEP amplitude 3 mins | STANDARD 12 single-pulse magnetic stimuli at 120-130% MT over the scalp area giving the most intense, reliable, MEP response. ALTERNATIVE 12 single-pulse magnetic stimuli at an intensity that elicits a 1 mV MEP NOTE: the first two pulses of TMS are often left out of analysis (some authors only ignore the first pulse), because of the possible acoustic or physical startle response to stimuli. | Global corticospinal excitability: Status of the excitatory horizontal axons, corticospinal neurons, spinal motoneurons, and muscle fibers, as well as the efficiency of the excitatory synapses connecting them. | STANDARD Ref ALTERNATIVE Ref NOTE Ref |
| Resting (rMT) motor threshold 4 mins | STANDARD Smallest intensity of maximum stimulator output (MSO) required to elicit MEPs of ≥ 50 μV in at least 5 out of 10 trials when muscle is completely relaxed. | - Neuron membrane excitability. - Number of neurons near threshold. | STANDARD Rossini & Rossi, 1998 |
| Active (aMT) motor threshold 6 mins | STANDARD The minimum stimulator output that induced MEPs of ~100 μV in at least 5 out of 10 consecutive trials during voluntary contractions of 5-10% maximum. See volitional strength section below. ALTERNATIVE The minimum stimulator output that induced MEPs of ~100 μV in at least 5 out of 10 consecutive trials during “light” voluntary contractions. NOTE: The alternative is very subjective without the ability to standardize the amount of contraction for each trial. It requires more repetitions and a lot of offline analysis to group(include/exclude) responses based on background EMG pre-stimulus (usually 50 – 100 ms window or pre-stimulus background EMG). | - Neuron membrane excitability. - Number of neurons near threshold. | STANDARD Rossini & Rossi, 1998 ALTERNATIVE Ref NOTE Ref |
| Recruitment curve (RC) a.k.a. Stimulus–response curves (SRC) 6 mins | STANDARD Multiple intensities of 4%-10% increments, starting from an intensity that does not elicit a response, to one of the following: 1) a plateau in MEP amplitude 2) subject discomfort 3) > 90% MSO or 200% rMT as per written protocol ALTERNATIVE 1 Multiple intensities of 5%-10% increments, between 80 to 200 % rMT. Collected individually (separate files) or randomly (one file). ALTERNATIVE 2 Program to randomize the order of intensities so responses are not recruited in a linear fashion. TIP: Best expressed in terms of the percentage of the muscle response amplitude evoked by supramaximal peripheral nerve stimulation of the target muscle. NOTE: A sigmoidal equation is then fitted to the data, which provides four key parameters including minimum, slope, stimulus intensity at which MEP amplitude is 50% of maximum (V50), and plateau value (maximum). | Global corticospinal excitability: Status of the excitatory horizontal axons, corticospinal neurons, spinal motoneurons, and muscle fibers, as well as the efficiency of the excitatory synapses connecting them. | STANDARD Ref [Past study = HAR-19-57] ALTERNATIVE 1 Ref ALTERNATIVE 2 Ref TIP Ref NOTE Ref |

| INTRACORTICAL EXCITABILITY |  |  |  |
| --- | --- | --- | --- |
| Measure | How | Reflects | References |
| Long intracortical inhibition (LICI) 6 mins | STANDARD Paired: 12 suprathreshold conditioning stimulation (CS) of 120% rMT up to 200 ms (e.g. 50, 100, 150 and 200 ms) before a suprathreshold test stimulation (TS) of 120% rMT over M1. Single: Compared to 24 unconditioned TS at 120% rMT. Usually a minimum of 2 12 unconditioned collections. More if the control is very variable. | Long duration GABAb-ergic inhibition of I waves. Essentially the same effect as CSP. | STANDARD Valls-Soléet al. 1992; Wassermann et al. 1996 |
| Intracortical facilitation (ICF) 9 mins | STANDARD Paired: 12 subthreshold CS of 80% rMT 6–25 ms (e.g. 10, 15, 20 and 25 ms) before a suprathreshold TS of 120% rMT. Single: Compared to the same 24 unconditioned TS at 120% rMT (see LICI). | Cortical net facilitation involving glutamate. | STANDARD Kujirai et al. 1993; Ziemann et al. 1996 |
| Short intracortical inhibition (SICI) 6 mins | STANDARD Paired: 12 subthreshold CS of 80% rMT up to 5 ms (e.g. 1 and 3 ms) before a suprathreshold TS of 120% rMT. Single: Compared to the same 24 unconditioned TS at 120% rMT (see LICI). A 2 min rest period should follow strong inhibition as this may affect the next collection. | Short duration GABAa-ergic inhibition. Likely mediated by the α2- or α 3-subunit of the GABAa receptor. | STANDARD Kujirai et al. 1993; Ziemann et al. 1996 |
| Short Intracortical facilitation (SICF) 6 mins | STANDARD Paired: 12 suprathreshold CS of 100-120% rMT at ISIs of 1.1–1.5 ms, 2.3–2.9 ms and 4.1–4.4 ms (e.g. 1.5, 2.7, and 4.5 ms) before a subthreshold TS of 90% rMT. Single: Compared to the same 24 unconditioned TS at 120% rMT (see LICI). | Targets I-wave generating circuits via excitatory interneuron interactions on the corticospinal tract. Facilitation is driven by glutamate/GABAa balance and modulated by dopamine. Intervals of ~ 1.5 ms between the facilitatory peaks closely matches the latencies between successive I-waves in epidural spinal cord recordings. Therefore, by analogy, it was suggested that SICF reflects facilitatory I-wave interaction. | STANDARD Tokimura et al. (1996) Ziemann et al. (1998) Chen and Garg (2000) Ziemann, U. I-waves in motor cortex revisited. Exp Brain Res 238, 1601–1610 (2020). https://doi.org/10.1007/s00221-020-05764-4 |

| SPINAL EXCITABILITY (SCS, TSS, TSCS, CES…) |  |  |  |
| --- | --- | --- | --- |
| Measure | How | Reflects | References |
| Recruitment curve (RC) a.k.a. Stimulus–response curves (SRC) 5 mins | STANDARD Multiple intensities of 5 mA – 25 mA increments, starting from an intensity that does not elicit a response, to one of the following: 1) a plateau in TEP amplitude 2) subject discomfort 3) e.g., 200% rMT as per written protocol ALTERNATIVE 1 Multiple intensities of 5%-10% increments, between 80 to 200 % rMT. ALTERNATIVE 2 Program to randomize the order of intensities so responses are not recruited in a linear fashion. TIP: Best expressed in terms of the percentage of the muscle response amplitude evoked by supramaximal peripheral nerve stimulation of the target muscle. NOTE: A sigmoidal equation is then fitted to the data, which provides four key parameters including minimum, slope, stimulus intensity at which TEP amplitude is 50% of maximum (V50), and plateau value (maximum). | Global corticospinal excitability: Status of the excitatory horizontal axons, corticospinal neurons, spinal motoneurons, and muscle fibers, as well as the efficiency of the excitatory synapses connecting them. | STANDARD Murray & Knikou, 2017; Murray & Knikou, 2019c; Knikou & Murray, 2019 Murray et al 2025 ALTERNATIVE 1 Ref ALTERNATIVE 2 Ref TIP Ref NOTE Ref |
| Homosynaptic depression (HD) Or Homosynaptic post-activation depression (PAD) phenomenon TBC 9 mins | STANDARD Single: 15 suprathreshold unconditioned TS of 120% rMT at different frequencies (e.g. 10, 8, 5, 3, 1 seconds). Paired: 15 suprathreshold stimulation of 120% rMT at different ISIs (e.g. 500, 300, 100, 60 milliseconds) at a constant rate of 0.2 Hz. ALTERNATIVE 1 Instead of 120% rMT as described above, 1 mV MEP size is used with the same frequencies/ISIs. | Underlying spinal neuronal plasticity and spinal inhibitory mechanisms. | STANDARD Murray & Knikou, 2017; Murray & Knikou, 2019c; Knikou & Murray, 2019 |

| MOTOR NERVE |  |  |  |
| --- | --- | --- | --- |
| Measure | How | Reflects | References |
| M-wave amplitude (Mmax) 2 mins | STANDARD Stimulation increased until no H-reflex is identified in the EMG recording and the M-wave reaches its maximal amplitude. 5 Mmax responses recorded. | Represents activation of the entire MN pool and, therefore, maximum muscle activation. | Pierrot-Deseilligny E, Mazevet D. The monosynaptic reflex: a tool to investigate motor control in humans: interest and limits. Neurophysiol Clin. 2000;30:67–80. Hugon M. Methodology of the Hoffmann reflex in man. In: Desmedt JE, editor. New Developments in Electromyography and Clinical Neurophysiology. New York, NY: Karger; 1973. pp. 277–293. OTHER Zehr EP. Considerations for use of the Hoffmann reflex in exercise studies. Eur J Appl Physiol. 2002;86:455–468. |
| H-Reflex threshold | TBC Note: Stimulation cannot evoke a response in all muscles. See REF for a great methodology paper on PNS. |  | NOTE Burke D. Clinical uses of H reflexes of upper and lower limb muscles. Clin Neurophysiol Pract. 2016 Apr 7;1:9-17. doi: 10.1016/j.cnp.2016.02.003. PMID: 30214954; PMCID: PMC6123946. |
| H-Reflex max | TBC | Hmax is a measure of maximal reflex activation or, stated differently, is an estimate of the number of MNs one is capable of activating in a given state. The Hmax/Mmax ratio can be interpreted as the proportion of the entire MN pool capable of being recruited. | OTHER Misiaszek JE. The H-reflex as a tool in neurophysiology: its limitations and uses in understanding nervous system function. Muscle Nerve. 2003 Aug;28(2):144-60. doi: 10.1002/mus.10372. PMID: 12872318. Palmieri RM, Ingersoll CD, Hoffman MA. The hoffmann reflex: methodologic considerations and applications for use in sports medicine and athletic training research. J Athl Train. 2004 Jul;39(3):268-77. PMID: 16558683; PMCID: PMC522151. |
| Recruitment curve (RC) a.k.a. Stimulus–response curves (SRC) 5 mins | STANDARD Stimulation intensity sequentially increased (typically by 0.05 mA increments) from a sub-H-reflex threshold intensity to a point when the H-reflex amplitude begins to decline. Thereafter, increased current intensity at a faster rate (0.2–1 mA) until a maximum M-wave (Mmax) is elicited. ALTERNATIVE Program … NOTE: 40 stimuli are bare minimum for creating a curve. However, the technique is very sensitive to changes in position and one might accidently start recruiting multiple curves is certain aspects are not well controlled for. |  | Roche N, Lackmy A, Achache V, Bussel B, Katz R. Impact of transcranial direct current stimulation on spinal network excitability in humans. J Physiol. 2009;587(Pt 23):5653-5664. Devanne H, Cohen LG, Kouchtir-Devanne N, Capaday C. Integrated motor cortical control of task-related muscles during pointing in humans. J Neurophysiol. 2002;87(6):3006-3017. Klimstra M, Zehr EP. A sigmoid function is the best fit for the ascending limb of the Hoffmann reflex recruitment curve. Exp Brain Res. 2008;186(1):93-105. NOTE Ref |
| Homosynaptic depression (HD) Or Homosynaptic post-activation depression (HPAD) phenomenon TBC 8 mins | - Single: 15 pulses elicited at various frequencies: 0.1 Hz, 0.2 Hz, 0.3 Hz, 1 Hz, 2 Hz, 5 Hz, 10 Hz and again at 0.1 Hz. The second 0.1 Hz bout, denoted “0.1 s”, verifies that the H-reflexes returned to the original amplitude, confirming that the electrode did not move. The first (non-depressed) H-reflex response is excluded from analysis. The mean amplitude of H-reflexes 2–15 at each frequency is then normalized to the mean value of the 0.1 Hz condition. H-reflexes recruited at an amplitude of 20–40% of Mmax are usually preceded by an M-wave. M-wave stability is further scrutinized to verify nerve activation differences did not underlie apparent H-reflex depression. 0.1 Hz conditions normalized to Mmax. - Paired: 5 doublets at ISIs of 20, 40, 60, 80, 100, 120, 150, 200, 250, 300, 500, 1000, 2000 and 5000 ms. The stimulating pairs were elicited in a randomized order with at least 10 s between each set. NOTE: In order to differentiate between the depression attributable to presynaptic inhibition, to post-activation depression or to joint effects of the two see REF. | Underlying spinal neuronal plasticity and spinal inhibitory mechanisms. Presynaptic inhibition is associated with a potent depolarization of afferent fibres (primary afferent depolarization; PAD) Post-activation depression occurs because the release of transmitter from Ia terminals following their activation is reduced for several seconds (Hultborn et al. 1996). | Crone C, Nielsen J. Methodological implications of the post activation depression of the soleus H-reflex in man. Exp Brain Res. 1989;78(1):28-32. Shields et al., 2011 Schindler-Ivens S, Shields RK. Low frequency depression of H-reflexes in humans with acute and chronic spinal-cord injury. Exp Brain Res. 2000;133(2):233-241. Chang et al., 2020 Hofstoetter US, Freundl B, Binder H, Minassian K (2019) Recovery cycles of posterior root-muscle reflexes evoked by transcutaneous spinal cord stimulation and of the H reflex in individuals with intact and injured spinal cord. PLoS ONE 14(12): e0227057. NOTE Kaczmarek D, Ristikankare J, Jankowska E. Does trans-spinal and local DC polarization affect presynaptic inhibition and post-activation depression? J Physiol. 2017 Mar 1;595(5):1743-1761. doi: 10.1113/JP272902. Epub 2017 Jan 17. PMID: 27891626; PMCID: PMC5330874. |
| Reciprocal inhibition |  |  |  |

FCR: stimulation of the median nerve through a bipolar electrode placed 2-3 cm below the elbow in the medial side of the arm (rectangular pulse of 0.5-1ms duration, 0.33 Hz). Note: a marked increase in the H-reflex during wrist flexion but not during pure pronation or finger flexion (criterion indicating that the reflex originated mainly from FCR).

ECR: stimulation of the radial nerve through a bipolar electrode placed about 10 cm above the elbow at an optimal point in the spiral groove (rectangular pulse of 1ms duration, 0.33 Hz). Note: M wave and H reflex of the ECR overlap, carefully measure respective recruitment curves by small increments of stimulus intensity to distinguish between the two potential waveforms.

| VOLITIONAL STRENGTH |  |  |  |
| --- | --- | --- | --- |
| Measure | How | Reflects | References |
| Wrist extension and flexion maximal voluntary contraction (MVC) 5 mins | A maximal contraction held for 5 s before releasing. Participants perform three trials, separated by at least 30-60 s of rest, until peak forces from two trials agreed within 5%. The greater of these two values is considered the MVC force. A 5 min rest period should follow the MVC measurements. Note: ALS might not be able to manage such a long hold so alternative methods might be performed. E.g. 2 s hold or instructing the participant to let go when they think they reached their max. | Determine maximal force. Assessing forearm muscle imbalances, fatigue susceptibility, and recovery. | ref |
| Pinch MVC | As per wrist MVC Precision (tip-to-tip) pinch. Tip of the thumb meets the tip of the index finger. Precision (pad-to-pad) pinch. Pad (pulp) of the thumb meets the pad of the index finger. Lateral key pinch. Pad of the thumb presses against the lateral (radial) side of the index finger. | Tip: Highest precision; used for picking up tiny objects like a pin or needle. Pad: General precision; used for zipping a jacket or picking up small beads. Key: Strongest pinch; used for turning a key or pulling up a zipper. | ref |
| Grip MVC | As per wrist MVC Power Grip (Grasp): Involves the whole hand wrapping around an object (like a handle or ball) and pressing it against the palm and thumb. It is used for strength-based tasks like holding a hammer. Precision Grip (Pinch): Involves the tips or pads of the fingers and thumb to manipulate small objects. This is used for dexterity-based tasks like holding a needle. | Reflects the synchronized activation of motor units; the brain often limits 100% recruitment as a protective mechanism. |  |